Application of Electrocoagulation in Various Wastewater and

Leachate Treatment – A Review

Laila Wahidah Mohamad Zailani1, Nur Shaylinda Mohd Zin1

1Department of Water and Environmental Engineering

Universiti Tun Hussein Onn Malaysia, Batu Pahat 86400, Johor.

Received XX XXXX 201X ; accepted XX XXXX 201X, available XX XXXX 201X

Abstract: Electrocoagulation is a method that has a great ability on various wastewater and leachate treatment. It has a potential in removing various pollutants such as chemical oxygen demand, turbidity, ammonia, color, and suspended solid. The effectiveness of electrocoagulation method depends on several factors such as electrode, current density, operation time and pH. The aim of this paper is to review the relevant literature that publishes from 2000 to 2015 on the factor that influence Electrocoagulation (EC). The review describes, discussing and compare the factors that influence the EC process in various wastewater and leachate treatment.

Keywords: physical-chemical treatment, electrocoagulation, wastewater, leachate

- Introduction

One of the major problems facing by the world today is to provide clean water for human population. The increasing of human population day by day give a shock matter   due to the corresponding improvement in the total quantity of industrial and municipal solid waste [1] [2].  According to Mogens Henze and Yves Comeau  [3], generally there are two type of wastewater which are generated  from society (domestic wastewater, wastewater from institutions, industrial wastewater, infiltration into sewers, storm water, leachate, septic tank wastewater )  and  wastewater generated internally in treatment plants such as (thickener supernatant, digester supernatant, reject water from sludge dewatering, drainage water from sludge drying beds, filter wash water and equipment cleaning water). According to previous study, treatment methods that have been used widely around the world to treat wastewater are biological and physical-chemical process [4]. Biological process is a process that uses a natural process which is rely on bacteria, nematodes, small organism. While physical- chemical process required additive of chemical in it process. One of the physical –chemical process that not required any additive of chemical is electrocoagulation (EC). EC is a unique method for water and waste water treatment and had been tested on various industrial effluent such as food industries, tannery, mechanical workshop (soluble oil), polymerization manufacture and wastewater textile industry [5].

According to Thakur and Chauhan [6], in an EC process, the coagulating   ion mostly generated continuously, and it consist of a few consecutive steps. First step is the coagulation formation by electrolytic oxidation of the sacrificial anode, second steps are the destabilization of the pollutant, particulate suspension, and emulsion breaking and for the last steps is an aggregation of the destabilized phase to form floc. Nowadays, EC method has been commercial, and it shows a better benefit compare to current treatment process in terms of equipment simplicity, portable, clear, colorless and odorless treated product [7]. Other advantages of EC are less treatment time, reduction of chemical   addition, ease of operation, rapid sedimentation of electro generated flocs and less amount of sludge production. Thus, EC provides more advantages compared to others coagulation methods. The review is basically a discussion on EC studies conducted from 2000 until 2015.

.

- Electrocoagulation

Electro-coagulation process has been practiced since the 20th century with limited success [8]. The successful application is very limited at that time. Basically, electrocoagulation process is a process to form a floc with the help of oxidized metal in wastewater where it needs to be cleaned by electro-dissolution of soluble anode [9]. Electrocoagulation can also define as a process to remove the suspended solid from wastewater by using electricity to neutralize the negative particles by the formation of hydroxide complexes in water to gather the suspended solid, help bridge, bind and strengthen the floc for sedimentation due to gravity force [10]. This process gathers the suspended solid in water without coagulant, the coagulation happened when the direct current was applied it capable in removing the small particles, and setting them into motion [11].

Several factors that influence the efficiency of electrocoagulation process are type of electrodes, gap between electrodes, number of electrodes, size of electrodes, configuration of metals, current density, charge loading, pH of sample, addition of supporting electrolyte and operation time [12]. The electrodes that are usually used are iron, aluminum and stainless steel. This pairs of metal sheets are known as anode and cathode [13]. During the electrocoagulation process cathode is oxidized (loses electrons) and the water is reduced (gains electrons) resulted in treated water and easy to settle floc [10].

  - Electrocoagulation Process

Basically, EC is an electrolysis process for wastewater treatment that using pair of metal sheet (electrodes) and arranged by pair of two -anode and cathodes [5]. Electrodes used in electrocoagulation process can be arranged in monopolar or bipolar. Electrochemical mechanism in aqueous solution is quite complex [14]. Generally, this process involves three types of mechanisms of electrocoagulation, electro-flotation and electro-oxidation [15]. The use of different electrode gives variety effect to the removing mechanism such as pH, density, COD, ammonia, color, suspended solid(SS). Electrode produced coagulant in the water sample are made from either aluminum or iron. Aluminum and iron cations dissolve from the anode according to Eqn 1 and 2 and cathode according to Eqn 3 [16].  Positive charged are attracted with the negative charged hydroxides in this solution and it produces ionic hydroxides that make a stronger attraction toward dispersed particle as the counter ions cause coagulation [17]:

Fe(s)–Fen+(aq)+ne-                                                         (1)                                                                                                  Al(s)–Al3+(aq)+3e-                                                                                         (2)

2H2O+2e-H2+2OH-                                                        (3)

Fig 1 : Principle electrocagulation

The principle of EC is shown in Figure 1. Metals adsorbed on the anode are useful for generating ions continuously to the anode surface. This will generate ferric hydroxide, and it will follow by a buildup of negative colloidal charged through an electric field near the anode [18]. Then these particles will be connected with iron hydroxide and indirectly eliminated through electrostatic attraction [8]. The electric field potential of colloidal particles to coagulate is higher than through the use of chemicals. Thus, the electrocoagulation process increases the rate of coagulation [19]. Treatment systems using electrocoagulation method will be more complete if followed with sedimentation and filtration process [20].

- Application of Electrocoagulation

3.1 Wastewater treatment

Table 1, show application of EC on various wastewaters such as wastewater from distribution network, restaurant wastewater rich with oil and grease, textile wastewater consists of small suspended particle, synthetics wastewater from factory and potato chips wastewater. Electrocoagulation process was run on different type of electrode to determine the efficiency removal. Several studies had investigated the factor affecting the performances of EC such as the type of electrode.

A recent study by previous researchers have stated that using aluminum and iron electrode are the best choice because of they are cheap and easy to find [21]. In this study, the researcher found that aluminum (Al) electrode give a better performance compare to iron (Fe) electrode. Furthermore, aluminum electrode does not give effect of color to supernatant. However, different researchers stated that iron electrode has more efficiency in nitrate removal than aluminum electrode [22]. The researcher found that high density of ions hydroxide produces from iron electrode give the advantages in removing of nitrate. However, advantages of iron electrode are imparting. Several researchers had studied the effect of current density. Malakootian et.al [22], reported that increasing electrical potential will increase the nitrate removal performances and it will produce higher floc in high voltage. This is in line with Kobya study. Kobya et.al [21]   found that current density gives a huge effect on COD removal and turbidity when the charge loading is increased. The percentage removal efficiency increased from 38% to 62% for COD and 56% to 98% turbidity. As the electrode and energy consumption increased higher performance of EC was recorded especially for aluminum electrode. The higher the voltages and charge loading applied in EC process the bigger energy consumption wills occur.

Adhoumm et.al [23] stated that current density factor gives an effect on the coagulant dosage rate and the production of bubble and size produced. These processes significantly affecting the performances of EC. It can be concluded that with higher current density, the bubble density become high, the floc increased and resulted in removal of the coagulant by flotation. According to Bazrafshan et.al [24], when applied voltage from 10 V to 40 V the COD and BOD5 removal slightly increase to 99%. Based on past researcher, current density basically one of the factor effecting the performances of EC.

According to Malakootian et.al [22], pH is also one of the factor affecting the EC performances. Chen et.al [25] stated   pH did not affect the removal efficiency significantly in term different type of wastewater. Vice versa, Kobya et.al [21], found out that pH playing important role in the EC process. It stated that when pH is higher than 6, removal efficiency of COD decreased. Adhoum et.al [23] recorded the removal efficiency of copper, zinc reach high value when pH is exceeding 4 and at the same time condition removal of chromium reached at maximum value about 83%. From the previous review show that the pH factor influenced the EC performances and the best pH value are indicated at alkali condition.

Table 1: Application by electrocoagulation process of various wastewater

| Sample | Current density | Operation time (min) | Dose coagulant (mg/L) | Electrodes/ Chemical coagulant | Percentage removal (%) |  | Author |
| --- | --- | --- | --- | --- | --- | --- | --- |
| synthetic (pH 4) | 40A/cm2 | 15 | NA | aluminum electrodes | 99 (Cu) | 83 (Cr) | Adhoum et al. (2004) |
| potato chips (pH 5) | 20 mA/cm2 | 30 | NA | aluminum electrodes | 60 (COD) | 98 (ss) | Kobya et al. (2006) |
|  | 20 mA/cm2 | 30 | NA | iron electrodes | 50 (COD) | 80 (ss) |  |
| textile water (EC pH 6.9) (CC pH 5.5) | 10 mA/cm2 | 20 | NA | aluminum electrodes | 63 (COD) | NA | Can et al. (2005) |
|  | 10 mA/cm2 | 10 | 0.8 Kg/m3 | aluminum electrodes + polyaluminium chloride | 80 (COD) | NA |  |
|  | 10 mA/cm2 | 10 | 0.8 | aluminum electrodes + alum | 65 (COD) | NA |  |
|  | NA | NA | 0.32 | polyaluminium chloride | 78 (COD) | NA |  |
|  | NA | NA | 0.32 | alum | 72 (COD) | NA |  |
| slaughterhouse (Ph 7.31) | 40V | 60 | 25 | aluminum electrodes + polyaluminium chloride | 80 | 50 | Bazrafshan et al. (2012) |
|  | NA | NA | 25 | polyaluminium chloride | 37 (COD) | 47 (TSS) |  |
| distribution network (pH 10) | 24A/m2 | 60 | NA | iron electrodes | 98 (calcium) | 97 (total hardness) | Malakootian et.al (2010) |
| Restaurant (pH 6-10) | 30-80 A/m2 | NA | NA | aluminum electrodes | 90 (COD) | 94 (oil and grease) | Chen et.al (2000), |

Table 2: Application by electrocoagulation process of leachate sample

| Sample | Current density | Operation time (min) | Dose coagulant (mg/L) | Electrodes/ Chemical coagulant | Percentage removal (%) |  | Author |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Leachate pH 4 | 10V | 100 | NA | aluminum electrodes | 74 (COD) | NA | Jotin et al. (2012) |
| Leachate Ph 5 | 75 mA/cm2 | NA | NA | aluminum electrodes | 48 (COD) | NA | Tezcan and Adunco ( 2014) |
| Leachate (pH 5.8) | 9V | 35 | NA | aluminum electrodes | 96 (COD) | 97 (turbidity) | Shivayogimath and Watawati (2013) |
| Leachate (pH 6) | 60 mA/cm2 | 30 | NA | iron electrodes | 81 (COD) | 72 (colour) | Moayerikashani et al. ( 2012a) |
| leachate (pH 6.5) | 2.98 mA/cm2 | 30 | NA | iron electrodes | 33 (COD) | 25 (ammonia) | Li et al. (2011) |
|  | 2.98 mA/cm2 | 30 | NA | aluminum electrodes | 21 (COD) | 20 (ammonia) |  |
|  | 4.96 mA/cm2 | 90 | 2319 | iron electrodes + sodium chloride | 93 (COD) | 39 (ammonia) |  |
| Leachate (pH 8.65) | 10V | 120 | 1.5%(w/v) | charcoal composite electrodes (C70-PVC30) + sodium chloride | 82 (COD) | 69 (ammonia) | Jumaah and Othman (2015) |
| leachate (pH8.2) | 34.8 mA/cm2 | 30 | NA | aluminum electrodes | 59 (COD) | NA | Ilhan et al. (2008), |
|  |  |  |  | iron electrodes | 33 (COD) | NA |  |
|  | NA | NA | 1500 | Alum | 31 (COD) | NA |  |

3.2 Leachate treatment

Table 2 shows the percentage removal of leachate sample by electrocoagulation process with different type of electrodes, supporting electrolyte to increase current conductivity and addition of chemical or natural coagulant to enhance formation of floc. Several authors were compared the percentage removal by electrocoagulation process with coagulation and flocculation process. In the treatment of leachate by EC, the main factor is the current density.

According   to Ilhan et.al [4], current density effecting the COD removal efficiency. The result shows the treatment changed with the increasing of current density from 348 to 631 A/m2. The COD removal efficiency obtained from this research increasing from 45.5% to 59%. Li et.al [25] show the removal efficiency of COD increasing from 28% to 93% as current density increased from 1.98 mA/cm2 to 4.96 mA/cm2. Moayerikashani et.al [26] stated that with current density 60 mA/cm2 the optimum reduction of COD removal is 81%. Moayerikashani et al [26] also conclude that the decreased current density resulted in reduction of COD removal. According to Shivayogimath and Watawati [27], the current density has affected the COD and turbidity removal. This proof that current density gives a huge effect to the removal of pollutant.

According to Lin et.al [25], pH factor has important rule in performance of the EC process.  The pH value mostly influent by the cathodes.  Increasing current density will affect the pH as well [4]. Referring to Li et.al [25], pH is the one of the major factor affecting the .al performances of EC. Based on Moayerikashani et, [26] studies, COD removal is 81% at pH 6 and when the pH value increases to acidic condition, the COD removal decreased.

However, Shivayogimath and Watawati [27] found out that, when the pH is in alkaline condition it removed removal 95.8% of COD and at pH 4 o 73.6% of COD was removed. By referring to Jotin.et al [28] maximum COD removal efficiency can be obtained at range  pH 4 until  pH 8 with 59% until 74%. Thus the best COD removal is at range pH 4-8. According to Ilhan et.al [4], iron and aluminum electrode have different performances in removing of COD. The result shows that aluminum gives effective performances than iron based on COD and NH3-N removals. According to Li et.al [25], different type of electrode gives different effect to the EC performances. Li et.al [25], shows that iron perform better than aluminum and the settlement of particle of iron are better than Al. By referring to Mansooreh et.al [29], stated that iron electrode is more economical in terms of energy consumption and suitable with economic evaluation. However, many researchers found that using aluminum electrode are more efficient than iron electrode and suitable to use in electrocoagulation application because it produce Al (III0)[30,31,32].

- Conclusion

Different electrodes materials resulted in different percentage removal COD, turbidity and ammonia. From the review, aluminum electrode performs better than iron electrode which is higher percentage removal was recorded in COD and suspended solid removal for various wastewater treatment. Moreover, for leachate treatment, aluminum electrode also shows a better efficiency in term of percentages removal efficiency than iron electrode on color, turbidity and COD. Current density is one of the major factor for EC performances. High current density produced a better removal of COD due the production of bubble and floc. Other than that pH factor is also considered as the important factor for EC performances. The activities of cathode also influent the pH value. Higher efficiency of EC can be obtained if current density, type of electrode and pH is considered in the treatment process.

- Acknowledgement

This work was supported by Universiti Tun Hussein Onn Malaysia under grant scheme FRGS (Vot 1570) and GPPS (U794).

References

[1] Bashir, M.J.K, Isa, M.H, Kutty, S.R.M, Awang,Z,Aziz , H.A, Mohajeri, S and Faroqi, I.H. (2009). Landfill leachate treatment by electrochemical oxidation. Waste Management,. Waste Management, 29, pp.2534-2541

[2]    Renou, S, Givaudan, J.G, Poulain, S,Dirassouyan, F and Moulin, P. (2008). Landfill leachate treatment: Review and opportunity. Journal Hazardous Materials, Vol.150 (3), pp.468-493

[3] Mogens Henze and Yves Comeau. (2008).       Wastewater Characterization . In Biological Wastewater Treatment: Principles Modelling and Design (pp. 34-35). Published by IWA Publishing, London, UK

[4]   Ilhan, F., Kurt, U., Apaydin, O., & Gonullu, M. T. (2008). Treatment of leachate by electrocoagulation using aluminum and iron electrodes. Journal of Hazardous Materials, 154(1–3), 381–389. 35

[5]    Butler, E., Hung, Y.-T., Yeh, R. Y.-L., & Suleiman Al Ahmad, M. (2011). Electrocoagulation in Wastewater Treatment. Water, 3(4), 495–525.

[6]    Thakur, S., & Chauhan, M. S. (2016). Treatment of Wastewater by Electro coagulation : A Review, 5(3), 104–110.

[7]    Labanowski, J., Pallier, V., & Feuillade-cathalifaud, G.(2010). Study of organic matter during coagulation and electrocoagulation processes : Application to a stabilized landfill leachate. Journal of Hazardous Materials, 179(1–3), 166–172.

[8]    Daneshvar, N., Sorkhabi, H. A., Kasiri,, M. (2012). M.B(2004).Decolorization of dye solution Acid Red 14 by electrocoagulation with a comparative  investigation of different electrode connection, Journal of Hazardous material, B112,55

[9] Malakootian, M., & Yousefi, N. (2009a). the Efficiency of Electrocoagulation Process Using Aluminum Electrodes in Removal of Hardness From Water. Iranian Journal of Environmental Health Science and Engineering, 6(2), 131–136.

[10]  Fagnekar, N. A., & Mane, P. S. (2015). Removal  Of Turbidity Using Electrocoagulation, 1, 252–260.

[11]  Koohestanian, A., Hosseini, M., & Abbasian, Z. (2008). The Separation Method for Removing of Colloidal Particles from Raw Water, 4(2), 266–273.

[12] Can, O. T., Kobya, M., Demirbas, E., & Bayramoglu, M. (2006). Treatment of the textile wastewater by combined electrocoagulation. Chemosphere, 62(2), 181–187.

[13] Roopashree, G. B., & Lokesh, K. S. (2014). Comparative study of electrode material ( iron , aluminium and stainless steel ) for treatment of textile industry wastewater, 4(4), 519–531.

[14] Lin. S.H and Chang, C.C. (2000). Treatment of Landfill Leachate by Combined Electro - Fenton oxidation and Sequencing Batch Reactor Method. . Water Resource,, Vol.34, no.17, pp. 4243-4249

[15] Butler, J. E. et al. Evolution of electron transfer out of the cell: comparative genomics of six Geobacter genomes. BMC Genomics 11, 40 (2010)

[16] Naje, A. S., & Abbas, S. A. (2013). Electrocoagulation Technology in Wastewater Treatment : A Review of Methods and Applications, 3(11), 29–43.

[17] YadavLin. S.H and Chang, C.C. (2000). Treatment of Landfill Leachate by Combined Electro - Fenton oxidation and Sequencing Batch Reactor Method. . Water Resource,, Vol.34, no.17, pp. 4243-4249

[18]  Mollah, M. Y. A., Schennach, R., Parga, J. R., & Cocke, D. L. (2001). Electrocoagulation ( EC ) — science and applications, 84, 29–41.

[19]  Michael J. Matteson, Regina L. Dobson, Robert W. Glenn, Jr., Nagesh S. Kukunoor, William H. Waits III, Eric J. Clayfield, (1995). Electrocoagulation and separation of aqueous suspensions of ultrafine particles, Physicochemical and Engineering Aspects 104) 101-10

[22]  Rahmani , A. R., (2008). Removal of water turbidity by the electrocoagulation method, J. Res. Health Sci., 8(1): 18- 24.

[21] Kobya, M., Hiz, H., Senturk, E., Aydiner, C., & Demirbas, E. (2006). Treatment of potato chips manufacturing wastewater by electrocoagulation. Desalination, 190(1–3), 201–211.

[22] Malakootian, M., Mansoorian, H. J., & Moosazadeh, M. (2010). Performance evaluation of electrocoagulation process using iron-rod electrodes for removing hardness from drinking water. Desalination, 255(1–3), 67–71.

[23]  Adhoum, N., Monser, L., Bellakhal, N., & Belgaied, J. E. (2004). Treatment of electroplating wastewater containing Cu2+, Zn 2+ and Cr(VI) by electrocoagulation. Journal of Hazardous Materials, 112(3), 207–213.

[24]  Bazrafshan, E., Mostafapoor, F., Soori, M. M., & Mahvi, A. H. (2012). Application of combined chemical coagulation and electro-coagulation process for carwash wastewater treatment. Fresenius Environmental Bulletin, 21(9 A), 2694–2701

[25] Chen.M. l, Lin SH, Treatment of textile wastewater by chemical methods for reuse. Water Res. 1997; 31:868–876

[26]  Li, X., Song, J., Guo, J., Wang, Z., & Feng, Q. (2011). Landfill leachate treatment using electrocoagulation. Procedia Environmental Sciences, 10(PART B), 1159–1164.

[27]  Moayerikashani, M., Soltani, S. M., Sobri, S., & Collection, A. S. (2012a). Treatment of a Malaysian Leachate Sample Using Electrocoagulation. International Journal of Chemical Engineering and Applications, 3(1), 1–4.

[28] Shivayogimath, C. B., & Watawati, C. (2013). Treatment of Solid Waste Leachate By Electrocoagulation Technology. International Journal of Research in Enginee Ring and Technology, 2319–232

[28] Jotin, R., Ibrahim, S., & Halimoon, N. (2012). Electro coagulation for removal of chemical oxygen demand in sanitary landfill leachate. International Journal of Environmental Sciences, 3(2), 921–930.

[29] Mansooreh Dehghani, Someih Shiebani Seresht ,Hassan Hashemi, "Treatment of hospital wastewater by electrocoagulation," International Journal of Environmental Health Engineering ,Vol. 2 , no. Issue 5, 2013.

[30] Bagherczadeh KM. A study of the removal of red dyes 14 mono Azo acid from contaminated water by electrocoagulation method.Thesis: Tabriz University; 2003.l Journal of Environmental Health Engineering ,Vol. 2 , no. Issue 5, 2013.

[31] Daneshvar N, Ashassi-Sorkhabi H, Tizpar A. Decolorization of orangeII by electrocoagulation method. Sep Purif Technol 2003;31:153-62

[32] Sengil IA, Ozacar M. Treatment of dairy wastewaters by electrocoagulationusing mild steel electrodes. J Hazard Mater 2006;137:1197-205..

Technology, 2319–2322.

[33]  Danial, R., Abdullaha, L. C., Mobarekeh, M. N., Sobri, S., & Adnan, N. M. (2015). A comparison between aluminium and iron electrodes in electrocoagulation process for glyphosate removal. Jurnal Teknologi, 77(32), 21–26.

[34] Donneys-Victoria, D., Marriaga-Cabrales, N., & Machuca-Martínez, F. (2014). 2 . Electrocoagulation for landfill leachate treatment : A review of patents and research articles. Evaluation of Electrochemical Reactors as a New Way to Environmental Protection, 661(100), 17–39.

[35] Emamjomeh, M. M., & Sivakumar, M. (2009). Review of pollutants removed by electrocoagulation and electrocoagulation/flotation processes. Journal of Environmental Management, 90(5), 1663–1679.

[36] Energetics, A. (2016). Treatment Of Landfill Leachate Effluent By Chemical Coagulation And Electrocoagulation : Without Correction Of Ph, 635–639.

[37]  EPA Victoria. (2009). Sampling and Analysis of Waters , Wastewaters , Soils and Wastes. Industrial Water Resourse Guidelines, 2009(June), 1–36.

[38]  Fernandes, A., Pacheco, M. J., Ciríaco, L., & Lopes, A. (2015). Review on the electrochemical processes for the treatment of sanitary landfill leachates: Present and future. Applied Catalysis B: Environmental, 176–177, 183–200.

[39]   Home, I., Science, M., Plant, E. T., Sulphate, F. A., Sulphate, M., Phthalate, P. H., … Tv, W. (2016). Chemical oxygen demand ( COD ) determination procedure and calculation, 10–11.

[40]   Inan, H., Dimoglo, A., Şimşek, H., & Karpuzcu, M. (2004). Olive oil mill wastewater treatment by means of electro-coagulation. Separation and Purification Technology, 36(1), 23–31.

[41]  Indore, S. G. S. I. T. S. (2014). Applications of Electrocoagulation in treatment of Industrial Wastewater : A Review, 3(11), 379–386.

[42]  Daud, Z., Fatihah, N., Hanafi, M., & Awang, H. (2013). Optimization of COD and Colour Removal From Landfill Leachate by Electro-Fenton Method. Australian Journal of Basic and Applied Science, 7(8), 263–268.

[43] Jumaah, M. A., & Othman, M. R. (2015). Optimization of Operating Conditions for Landfill Leachate Treatment Using Electrochemical Oxidation Technique. International Journal of ChemTech Research, 8(2), 783–787.

[44] Kabuk, H. A., Ilhan, F., Avsar, Y., Kurt, U., Apaydin, O., & Gonullu, M. T. (2014). Investigation of Leachate Treatment with Electrocoagulation and Optimization by Response Surface Methodology. Clean-Soil Air Water, 42(5), 571–577.

[45]   Katal, R., & Pahlavanzadeh, H. (2011). Influence of different combinations of aluminum and iron electrode on electrocoagulation efficiency: Application to the treatment of paper mill wastewater. Desalination, 265(1–3), 199–205.

[46] Khandegar, V., & Saroha, A. K. (2013). Electrocoagulation for the treatment of textile industry effluent - A review. Journal of Environmental Management, 128(July 2013), 949–963.

[47] Mahmad, M. K. N., Rozainy, M. A. Z. M. R., Abustan, I., & Baharun, N. (2016). Electrocoagulation Process by Using Aluminium and Stainless Steel Electrodes to Treat Total Chromium, Colour and Turbidity. Procedia Chemistry, 19, 681–686.

[48] Malakootian, M., & Yousefi, N. (2009b). The Efficiency Of Electrocoagulation Process Using Aluminum Electrodes IN REMOVAL OF Hardness From Water, 6(2), 131–136.

[49]  Malakootian, M., Yousefi, N., & Fatehizadeh, A. (2011). Survey efficiency of electrocoagulation on nitrate removal from aqueous solution, 8(1), 107–114.

[50]  Moayerikashani, M., Soltani, S. M., Sobri, S., & Collection, A. S. (2012b). Treatment of a Malaysian Leachate Sample Using Electrocoagulation, 3(1), 1–4.

[51] Moghaddam, A. (2011). A Comparison Between Aluminum and Iron Electrodes on Removal of Phosphate from Aqueous Solutions by Electrocoagulation Process, 5(2), 403–412.

[52]  Murthy, Z. V. P., Nancy, C., & Kant, A. (2007). Separation of Pollutants from Restaurant Wastewater by Electrocoagulation. Separation Science and Technology, 42(4), 819–833.

[53]  Ozturk, T., Veli, S., & Dimoglo,  a. (2013). The Effect of Seawater Conductivity on the Treatment of Leachate by Electrocoagulation. Chemical and Biochemical Engineering Quarterly, 27(3), 347–354.

[54]  Poveda, M., Yuan, Q., & Oleszkiewicz, J. (2016). The Effectiveness of Pretreatment Methods on COD and Ammonia Removal from Landfill Leachate. International Journal of Environmental Science and Development, 7(4), 257–262.

[55]  Raghab, S. M., Abd, A. M., Meguid, E., & Hegazi, H. A. (2013). Treatment of leachate from municipal solid waste landfill. HBRC Journal, 9(2), 187–192.

[56] Ricordel, C., & Djelal, H. (2014). Treatment of landfill leachate with high proportion of refractory materials by electrocoagulation: System performances and sludge settling characteristics. Journal of Environmental Chemical Engineering, 2(3), 1551–1557.

[57]  U., T. U., & E., O. (2014). Electrocoagulation of Landfill Leachate with Monopolar Aluminum Electrodes. Journal of Clean Energy Technologies, 2(1), 15–17.

[58]  Veps, M. (n.d.). Electrocoagulation in the treatment of industrial waters and wastewaters.